- Can you sent us a completed product picture or the previous company that you purchase from?

There is no such product yet. There is no previous company. We need to develop this product

- Do you have the function document?

We don’t have a functionality document yet. However, the basic functionality is quite simple: 30 ball bearings. Each one moves separately within the range of 0..200 micrometers. We need to build a multichannel proximity sensor (30 sensors – each one senses the movement of each ball bearing). Then the measurement from all 30 sensors are sent by SPI to our own processing board. Precision of each channel – 1 micrometer

- Sensor specification?

DRV 5053 or similar

- SPI communication protocol?

Yes. But the supplier can offer a different protocol. We need to transmit 30 channels – minimum 12 Bit each within 250 microseconds.

- The PCB mechanical size4.

PCB Size ： lenghth : 180mm,  width :10-15mm

Yes.

- Do you need the chip or module?

We need the PCB. From one side – Hall Sensors. From another side – the electronics. Probably, we will need a small additional PCB to be connected to OUR OWN electronic board: the RECEIVER.

- Is the measure result transmit through one road of SPI or 30 road?

Edward: We need 1 serial output. Every 250 microsecond we need to send a string:
(Channel1[12 Bit], Channel2[12 Bit],…..., Channel30[12Bit])