说明

- 要制作的ADSL猫的使用方法如附图，是石油测井中典型用法。功能是：地面将控制命令下发到井下各仪器，井下各仪器将数据上传到地面。通信猫要用两个，一个在地面，一个在井下，用电缆连接。工作时这样连接：计算机+交换机+制作的通信猫A+双绞线（10000米，电缆）+制作的通信猫B+以太网路由器数据发送网卡。工作过程如下。下行过程：计算机以TCP/IP协议发送命令数据，通过制作的通信猫A调制、驱动，经过双绞线传输，达到制作的通信猫B，由制作的通信猫B解调，发送网卡执行命令。上行过程：网卡采集的数据，以以太网发送至制作的通信猫B调制，驱动至双绞线传输，制作的通信猫A接收数据并解调，输入计算机。

- 试验板电路使用的元器件可以使用常温器件和功能更高器件，但该器件必须有耐高温性能的产品。因该制作的通信猫要工作在175度环境中。

- Pcb板宽度55mm，长度根据实际确定。

- 上一次发给你的英文资料制作的调制解调器是2000年以前产品，可能还没有专用芯片。这一次发给你的附图是现在使用的一个方案。 唯一问题是U7芯片耐温不够。

- 也可以使用其他方案，但要求能在175度工作，且AFE要采用DMT调制。

- 提供资料：原理图，PCB板图，源程序，器件清单，开发工具。

据我了解 用tc3162+tc3086制造的ADSL基本能达到要求。

附件2  ADSL板原理图

AEF模块原理图

Clock模块原理图

RS232模块原理图

CPU模块原理图

MⅡ—PHY模块原理图

MCU_interface原理图

memory模块原理图

Misc_Stat模块原理图

SOC—power原理图

ADSL板top器件布局

bottom器件布局

ADSL板元器件清单

| Bill of Materials |  |  | Bill of Materials For PCB Document [telemetry-02.PcbDoc] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Source Data From: |  | telemetry-02.PcbDoc |  |  |  |  |
| Project: |  | 580NTGR |  |  |  |  |
| Variant: |  | None |  |  |  |  |
| Creation Date: | 2016/1/14 | 10:24:18 |  |  |  |  |
| Print Date: | 31-Jul-17 | ####### |  |  |  |  |
| Designator | Footprint |  | Comment |  | Quantity |  |
| C1, C26, C39, C42, C73, C81 | SMD_CAP_B |  | 100uF |  | 6 | 黑色10uF代替 |
| C2, C4, C5, C6, C7, C8, C9, C10, C11, C12, C18, C19, C20, C21, C22, C24, C25, C27, C29, C30, C31, C32, C33, C34, C35, C36, C37, C38, C40, C43, C47, C48, C49, C50, C51, C52, C53, C54, C55, C57, C58, C87, C88, C89, C90, C91, C92, C93, C94, C95, C98, C101, C102, C103, C104, C105, C106, C107, C108, C109, C110, C111, C112, C113, C114, C115, C116, C117 | CR2012-0805 |  | 100nF |  | 68 |  |
| C3, C28, C41, C44, C46, C86, C99, C100 | CR2012-0805 |  | 10nF |  | 8 |  |
| C13 | CR2012-0805 |  | 1uF |  | 1 |  |
| C14 | CR2012-0805 |  | 33pF |  | 1 |  |
| C15, C23 | SMD_CAP_B |  | 10uF |  | 2 | 黑色 |
| C16, C17 | CR2012-0805 |  | 100pF |  | 2 |  |
| C45, C85, C96, C97 | SMD_CAP_B |  | 47uF |  | 4 | 黑色10uF代替 |
| C56 | 1812 |  | 1000pF/2KV |  | 1 |  |
| C59 | CR2012-0805 |  | 3n3 |  | 1 |  |
| C60, C118 | CR2012-0805 |  | 1nF |  | 2 |  |
| C61 | CR2012-0805 |  | 470pF |  | 1 |  |
| C62, C63 | CR2012-0805 |  | 18pF |  | 2 |  |
| C119 | CR2012-0805 |  | 10pF |  | 1 |  |
| D1, D3, D6, D8 | 0805DIODE |  | LED_RED |  | 4 |  |
| D2, D4 | 0805DIODE |  | LED_GRE |  | 2 |  |
| J1 | HDR2X10 |  | Header 10X2 |  | 1 |  |
| J2 | HDR2X5 |  | RJ45_232 |  | 1 |  |
| J3 | 17PIN |  | 17PIN |  | 1 |  |
| L1, L2 | 1210 |  | Inductor |  | 2 | 短接 |
| P1 | HDR1X7 |  | Header 7 |  | 1 |  |
| R1, R2, R5, R6, R7, R9, R10, R11, R19, R20, R22, R26, R36, R62 | CR2012-0805 |  | 10k |  | 14 |  |
| R3, R8, R25 | CR2012-0805 |  | 33R |  | 3 |  |
| R4 | CR2012-0805 |  | 0R |  | 1 | 不焊 |
| R12, R13, R63, R64, R65 | CR2012-0805 |  | 330 |  | 5 |  |
| R27, R28, R37, R39 | CR2012-0805 |  | 50R |  | 4 |  |
| R35 | CR2012-0805 |  | 4.7k |  | 1 |  |
| R40 | CR2012-0805 |  | 1k |  | 1 |  |
| R41 | CR2012-0805 |  | 75R |  | 1 |  |
| R42, R43, R44, R45 | CR2012-0805 |  | 43R |  | 4 |  |
| R46, R47 | CR2012-0805 |  | 1K3 |  | 2 |  |
| R48, R49 | CR2012-0805 |  | 750R |  | 2 |  |
| R50, R61 | CR2012-0805 |  | 2k2 |  | 2 |  |
| R51 | CR2012-0805 |  | 5k6 |  | 1 |  |
| R52 | CR2012-0805 |  | 12k7 |  | 1 |  |
| R53, R56 | CR2012-0805 |  | 47R |  | 2 |  |
| R54, R55 | CR2012-0805 |  | 2R4 |  | 2 |  |
| R57, R58 | CR2012-0805 |  | 3k3 |  | 2 |  |
| R71 | CR2012-0805 |  | 680 |  | 1 |  |
| RN1, RN23, RN24, RN25, RN30, RN31, RN32 | RNRES |  | 10k |  | 7 |  |
| RN13, RN14, RN26, RN27, RN33 | RNRES |  | 10KX4 |  | 5 |  |
| T1 | CL62183ES-G |  | CL62183ES-G |  | 1 | DTP308S |
| U1 | TQ144 |  | STR710FZ2T6 |  | 1 |  |
| U2 | SO-8 |  | SP708S |  | 1 |  |
| U3 | SP3232SOP16 |  | SP3232E |  | 1 |  |
| U4, U5 | TSSOP54 |  | CY7C1069AV33 |  | 2 |  |
| U6 | TSSOP48 |  | SST39VF6401 |  | 1 |  |
| U7 | BGA324X18 |  | PEF22618 |  | 1 |  |
| U8 | QUAD48 |  | AC101 |  | 1 |  |
| U9 | SOP16 |  | HS9016 |  | 1 |  |
| U10 | SO-8 |  | LC03-3.3 |  | 1 | 不焊 |
| X1 | XTAL |  | 16MHz |  | 1 |  |
| X2 | XTAL |  | 25MHz |  | 1 |  |
| Y1 | xtal2 |  | 32kHz |  | 1 |  |
| Y2 | XTAL1 |  | 20.48MHz |  | 1 |  |
|  |  |  |  |  | 188 |  |
| Approved |  | Notes |  |  |  |  |