# Phyton Emulator Pod Setup Guide for TUSB3200 EVM, TUSB3200 EVM2 and TAS1020 EVM

## Introduction

The purpose of this paper is to give the user instructions on setting up Phyton emulator to work with TUSB3200 EVM, TUSB3200 EVM2 and TAS1020 EVM.  Instruction on how to use the Phyton emulator is not the intention of this paper.  This paper assumes that the Phyton emulator software has been loaded and the emulator pod has been hooked up to the PC according to the manufacturer’s instructions.

Note:  The Phyton In-Circuit Emulator includes:  PICE-51, P051-RX/550, A051D40, and TC-51.

## TUSB3200 EVM

  - Jumper Settings:

    - JP4 in

    - JP15 out

    - Other jumpers – please consult TUSB3200 EVM User’s Guide

Note:  The intention of these settings is to switch the EVM from internal to external MCU mode.

  - Emulator pod hook up:

    - Insert the Phyton emulator pod to J3.  Make sure the pin orientation and pin alignment are correct.

  - Setting up the software:

      - On the emulator window, go to Configure->Hardware Configuration, then select:

-	Target processor Intel 8052.

-	Options:  Enable RST from the target board

### Allow reading of trace buffer at run-time

-	Use P3.6 pin as:  write control pin

-	Use P3.7 pin as:  read control pin

- Clock generator: From the target board

      - On the emulator window, go to Configure->Memory Map, then select:

-	Existing Ranges - Select Range 1 - Select Edit

-	Address Range: Start: 0F800H; End: 0FFFFH

-	Memory Type: Xdata Only

-	Map Range to:  Target

      - On the emulator window, go to file->load file for debugging:

-	Browse to Rev927 file (or the latest file) location and select it.

      - Press reset on the TUSB3200 EVM.  Assuming the EVM is powered up and running.

      - On the emulator window, click on Reset icon, and then click on run.

      - Plug the USB cable from the host computer to the TUSB3200 EVM.

Note:  The TUSB3200 should enumerate and stream audio data.

## TUSB3200 EVM2 (6-channel EVM)

  - Jumper Settings:

    - JP4 in

    - JP15 out

    - Other jumpers – please consult TUSB3200 EVM2 User’s Guide

Note:  The intention of these settings is to switch the EVM from internal to external MCU mode.

  - Emulator pod hook up:

    - Insert the Phyton emulator pod to J3.  Make sure the pin orientation and pin alignment are correct.

  - Setting up the software:

      - On the emulator window, go to Configure->Hardware Configuration, then select:

-	Target processor Intel 8052.

-	Options:  Enable RST from the target board

### Allow reading of trace buffer at run-time

-	Use P3.6 pin as:  write control pin

-	Use P3.7 pin as:  read control pin

- Clock generator: From the target board

      - On the emulator window, go to Configure->Memory Map, then select:

-	Existing Ranges - Select Range 1 - Select Edit

-	Address Range: Start: 0F800H; End: 0FFFFH

-	Memory Type: Xdata Only

-	Map Range to:  Target

      - On the emulator window, go to file->load file for debugging:

-	Browse to v1.01 file (or the latest file) location and select it.

      - Press reset on the TUSB3200 EVM2.  Assuming the EVM is powered up and running.

      - On the emulator window, click on Reset icon, and then click on run.

      - Plug the USB cable from the host computer to the TUSB3200 EVM2.

Note:  The TUSB3200 should enumerate and stream audio data.

## TAS1020 EVM

  - Jumper Settings:

    - JP3 to EXT

    - Other jumpers – please consult TAS1020 EVM User’s Guide

Note:  The intention of these settings is to switch the EVM from internal to external MCU mode.

  - Emulator pod hook up:

    - Insert the Phyton emulator pod to J3.  Make sure the pin orientation and pin alignment are correct.

  - Setting up the software:

      - On the emulator window, go to Configure->Hardware Configuration, then select:

-	Target processor Intel 8052.

-	Options:  Enable RST from the target board

### Allow reading of trace buffer at run-time

-	Use P3.6 pin as:  write control pin

-	Use P3.7 pin as:  read control pin

- Clock generator: From the target board

      - On the emulator window, go to Configure->Memory Map, then select:

-	Existing Ranges - Select Range 1 - Select Edit

-	Address Range: Start: 0F810H; End: 0FFFFH

-	Memory Type: Xdata Only

-	Map Range to:  Target

      - On the emulator window, go to file->load file for debugging:

-	Browse to where ROM file (for TAS1020A) is located and select it.

-	Then click on file to download file for debug again to download application file.  Browse to V17 file (or the latest file) location and select it. This applies for TAS1020A only. For the TUSB3200 you only have to download file for application.

      - Press reset on the TAS1020A EVM.  Assuming the EVM is powered up and running.

      - On the emulator window, click on Reset icon, and then click on run.

      - Plug the USB cable from the host computer to the TAS1020A EVM.

Note:  The TAS1020A should enumerate and stream audio data.