TAS1020B HEADER UTILITY USER’S GUIDE

Before using the TAS1020 HeaderGui.exe program you will need to do the following.  First, create a Hex file from your C code.  Then convert the Hex file to Binary using the Hex2Bin.exe program provided.  Accept all of the default settings.  This will create a binary image of your application code that is now ready to have a TAS1020B header added.

Using the TAS1020B HeaderGui.exe Program

1.  Introduction:

This program is used for creating the TAS1020B download/program images.

Usage:

A.   For menu GUI: headergui <RT>

B.   For help: headergui -h<RT>

C.   For command line: headergui -s <header setup file>; -i <application code file>; -o <output image file> <RT>

- header setup file: Text file contains information for creating the image header.

- application code file: binary file contains application code. This file is used for creating the application type image. For the device type image, the "-i" option is not required.

- output image file: binary output image file name.

- Instructions:

  - Double click on “headergui.exe”, a Utility window will be displayed.

  - Enter in the data required for header.  All value is in hexadecimal.

- Vendor ID (self explanatory)

- Product ID (self explanatory)

- Product Version (self explanatory)

- Firmware Version (self explanatory)

- String Support Check Box (self explanatory)

- Language ID (0409 for English)

- Company Name (self explanatory)

- Product Name (self explanatory)

- Device Max Power in 2 mA unit (e.g. 100 mA = 32H)

- Self Power Checkbox (self explanatory)

- Bus Power Checkbox (self explanatory)

- CPU Speed Select (self explanatory)

- I2C Speed Select (self explanatory)

- Header Setup File (Point to directory which contains header text file or type in directory and file name of the header text file to be saved)

- Save/Open (Click save when information is entered the first time or Open when previous header text file has been saved.)

- I2C Write Page Size (self explanatory)

- I2C Read Page Size (self explanatory)

- Input Application In Binary Format (Point to directory which contains application code in binary format.  This is the binary file that requires the header to be added.)

- EEPROM Type (If App is checked, the output file will have the application code plus the header; If Device is checked, the output file will just contain the header without the application code.  If your intention is to build a file with just the header and no application code, do not specify Input Application file.)

- Output File in binary format (Point to directory and type in file name which you want to store your output binary file.)

- Download Target (This field is here because it prevents you from selecting overwrite header bit with the intention of using it for RAM only.  If you select overwrite bit and there is not an EEPROM present, the device will hang.  Therefore, this field is just a reminder of the potential conflict.)

- EEPROM Header Overwrite (self explanatory)

- Create Image (click this when you are satisfied with all the entered information.)